# Assembly Examples

USMAN BASHARAT

00874782

TASK 0 - Using the grey IDC10 ribbon cable (DO NOT USE THE RAINBOW RIBBON CABLE)

- Check that all the boxes have the IDC connections with the the notch facing inwards, as shown for the switch light box diagram above. (Sometime these connections pull out and are incorrectly reinserted back to front)

- How to Test IDC cables. Connect the switch light box's switches to the switch light box's LED's. There should be a one to one relationship of the switch to the LED above it. If there is not, or some LEDs do not illuminate, then normally the cable is at fault.

- Connect the switch light box's switches to the 'Traffic Lights' interface box and note the relationship of switches to the traffic light's LEDs0020

- Connect the switch light box's switches to the 'Dual Seven Segment Display' interface box and note the relationship of switches to the seven segment display's segments.

- Connect the STK200's on board push buttons to the switch light box's LEDs and note the relationship of the on board buttons to the switch light box's LEDs.

- Connect the switch light box's switches to the STK200's on board LEDs and note the relationship of switches to the LEDs

Task 1

;Program to output 55 hexadecimal to ports B and port C

.equ    PORTA =$1B                 ;Port A Output Address

.equ    DDRA  =$1A                 ;Port A Data Direction Register Address

.equ    PINA  =$19                  ;Port A Input Address

.equ    PORTB =$18                 ;Port B Output Address

.equ    DDRB  =$17                 ;Port B Data Direction Register Address

.equ    PINB  =$16                 ;Port B Input Address

.equ    PORTC =$15                 ;Port C Output Address

.equ    DDRC  =$14                  ;Port C Data Direction Register Address

.equ    PINC  =$13                  ;Port C Input Address

.equ    PORTD =$12                 ;Port D Output Address

.equ    DDRD  =$11                 ;Port D Data Direction Register Address

.equ    PIND  =$10                 ;Port D Input Address

;Program Initialisation

;Initialise output ports

ldi   r16,$ff

out   DDRB,r16            ;Set Port B for output by sending $FF to direction register

out   DDRC,r16            ;Set Port C for output by sending $FF to direction register

;Main Program

;Output $55 to Port B and Port C

main:   ldi   r16,$F0            ;Load 55 hex into r16

out   PORTC,r16          ;Output 55 hex to Port C

out   PORTB,r16          ;Output 55 hex to Port B

rjmp  main               ;Repeat forever

Calculation

Just add all the clocks up as shown. Adding them equals towards 8

Task 2

;Program to echo the switches attached port A to ports B and C

;Program to echo the switches attached port A to ports B and C

;Port Addresses

.equ	PORTA	=$1B

.equ	DDRA	=$1A

.equ    PORTB  =$18

.equ    PINB   =$16

.equ	PORTC	=$15

.equ    PINC	=$13

.equ    PORTD	=$12

.equ	DDRD	=$11

;Register Definitions

.def     temp   =r16               ;Temporary storage register

;Program Initialisation

;Initialise Input Ports

ldi    temp,$00

out    PORTC,temp          ;Set Port A for input by sending $00 to direction register

out    PORTB,temp

; Initialise output ports

ldi    temp,$FF           ;Set Port C for output by sending $FF to direction register

out    DDRA,temp

out    DDRD,temp

; Main Program

loop:    in     r17,PINC

in     r18,PINB

out    PORTD,r17

out    PORTA, r18

rjmp   loop   Task 3

chase2.asm

;Program to sequence LEDs on port C with delay loop

;Port Addresses

.equ     PORTC  =$15              ;Port C Output Address

.equ     DDRC   =$14              ;Port C Data Direction Register

;Register Definitions

.def     leds   =r0               ;Register to store data for LEDs

.def     temp   =r16              ;Temporary storage register

;Program Initialisation

;Initialise output ports

ldi    temp,$ff

out    DDRC,temp         ;Set Port C for output by sending $FF to direction register

;Initialise Main Program

sec                      ;Set carry flag to 1

clc    leds              ;Clear LEDs

;Main Program

forever: out    PORTC,leds        ;Display LEDs to port C

;delay section of code (25.344 ms @ 1MHz) - utilises r25,r24

ldi    r24,$C4          ;Initialise 2nd loop counter

loop2:   ldi    r25,$FF          ;Initialise 1st loop counter

loop1:   dec         r25              ;Decrement the 1st loop counter

brne   loop1            ;and continue to decrement until 1st loop counter = 0

dec    r24               ;Decrement the 2nd loop counter

brne   loop2            ;If the 2nd loop counter is not equal to zero repeat the 1st loop, else continue

;end of delay section of code

rol    leds              ;Rotate LEDs left by 1 bit through carry flag

rjmp   forever

Calculation Delay

1+[(255 * 3) – 1] = 765 one loop

1+765*255 + (255-1)*3+2 = 195840 maximum delay possible using two nested loops

195840/100000 = 195.840ms delay

Task 4

Using three loops

1+(255-1)*3+2=765

1+195840*255+(255-1)*3+2 = 49939965

4993965/1000000 = 49.939965s so 49.94s

Delay for 1.175s

1+(255-1)*3+2=765

1+765*255+(255-1)*3+2 = 195840

1+195840*6+(6-1)*3+2 = 1175058s

1175058s/1000000 = 1.175058s so 1.175s

TASK 5- Attach the switch light box's LEDs to Port C.

- Calculate the maximum delay possible with this technique  @ 1 MHz clock frequency. (Answer 262.141 ms)

- Alter 'chase 4' so the delay is 150 ms.

Demonstrate the program to the tutor and upload the program, flow chart and delay calculations to your logbook.

(3*255)+(3*255*255)+(3*255*255*255)+(3*255*255*255*255) = 12734691840 micro seconds

150000/4=37500= $927C

;Program to sequence LEDs on port C using a delay subroutine

;Status Register Address

.equ     SREG   =$3F               ;Status Register Address

;Stack and Stack Pointer Addresses

.equ     SPH    =$3E              ;High Byte Stack Pointer Address

.equ     SPL    =$3D              ;Low Byte Stack Pointer Address

.equ     RAMEND =$25F             ;Stack Address

;Port Addresses

.equ     PORTC  =$15               ;Port C Output Address

.equ     DDRC   =$14              ;Port C Data Direction Register Address

;Register Definitions

.def     leds   =r0                ;Register to store data for LEDs

.def     temp   =r16               ;Temporary storage register

.def     save   =r19              ;Temporary storage register for status register

.def     YL     =r28              ;Define low byte of Y

.def     YH     =r29               ;Define high byte of Y

;Program Initialisation

;Set stack pointer to end of memory

ldi    temp,high(RAMEND)

out    SPH,temp          ;Load high byte of end of memory address

ldi    temp,low(RAMEND)

out    SPL,temp          ;Load low byte of end of memory address

;Initialise output ports

ldi    temp,$ff

out    DDRC,temp          ;Set Port C for output by sending $FF to direction register

;Initialise Main Program

sec                       ;Set carry flag to 1

clr    leds              ;Clear LEDs

;Main Program

forever: out    PORTC,leds        ;Display leds to port C

rcall  delay              ;Call delay subroutine

rol    leds              ;Rotate leds left by 1 bit through carry flag

rjmp   forever            ;Continue forever

;Delay Subroutine (25.351 ms)

delay:   in     save,SREG          ;Preserve status register

ldi    YH,high($927C)    ;Load high byte of Y

ldi    YL,low($927C)     ;Load low byte of Y

loop:    sbiw   Y,1               ;Decrement Y

brne   loop              ;and continue to decrement until Y=0

out    SREG,save          ;Restore Status register

ret                       ;Return

TASK 6- Attach the switch light box's LEDs to Port C and the switch light box's switches to Port A

- Alter 'chase 5' so instead of the poll being on the MSB on port A, make the poll the Least Significant Bit (LSB) on port A - i.e. switch 0 on the switch light box . Note - operating switches 7 to 1 on the switch light box should not affect the program. Hint - reflect on subnet masks from the networking course.

;Program to sequence LEDs on port C, uses the MSB on switches to change direction

;Stack and Stack Pointer Addresses

.equ     SPH    =$3E              ;High Byte Stack Pointer Address

.equ     SPL    =$3D              ;Low Byte Stack Pointer Address

.equ     RAMEND =$25F             ;Stack Address

;Port Addresses

.equ     DDRA   =$1A               ;Port A Data Direction Register Address

.equ     PINA   =$19               ;Port A Input Address

.equ     PORTC  =$15               ;Port C Output Address

.equ     DDRC   =$14               ;Port C Data Direction Register Address

;Register Definitions

.def     leds   =r0                ;Register to store data for LEDs

.def     temp   =r16               ;Temporary storage register

.def     chdir  =r20              ;Register determining sequence direction of LEDs

;Program Initialisation

;Set stack pointer to end of memory

ldi    temp,high(RAMEND)

out    SPH,temp          ;Load high byte of end of memory address

ldi    temp,low(RAMEND)

out    SPL,temp          ;Load low byte of end of memory address

;Initialise Input Ports

ldi    temp,$00

out    DDRA,temp          ;Set Port A for input by sending $00 to direction register

;Initialise Output Ports

ldi    temp,$ff

out    DDRC,temp         ;Set Port C for output by sending $FF to direction register

;Initialise Main Program

sec                      ;Set carry flag to 1

clr    leds              ;Clear leds

;Main Program

forever: out     PORTC,leds        ;Display leds to port C

rcall  delay             ;Call delay subroutine

rcall  pollswt           ;Poll switches to check direction change

bst    chdir, 0             ;Test if negative

brts   right             ;If switch 7= 1 chase right else if if switch 7 = 0 chase left

;Rotate leds left

left:     rol    leds              ;Rotate leds left by 1 bit through carry flag

rjmp   forever

;Rotate leds right

right:   ror    leds              ;Rotate leds right by 1 bit through carry flag

rjmp   forever

;Polling Subroutine

pollswt: in     chdir,PINA        ;Read switches on switch light box

ret

;delay section of code (25.348 ms @ 1MHz) - utilises r25,r24

delay:   ldi    r24,$21          ;Initialise 2nd loop counter

loop2:   ldi    r25,$FF          ;Initialise 1st loop counter

loop1:   dec         r25              ;Decrement the 1st loop counter

brne   loop1            ;and continue to decrement until 1st loop counter = 0

dec    r24               ;Decrement the 2nd loop counter

brne   loop2            ;If the 2nd loop counter is not equal to zero repeat the 1st loop, else continue

ret                   ;Return

Task 7

;Program to sequence LEDs on port C, uses interrupt on button SW2

;Stack and Stack Pointer Addresses

.equ     SPH    =$3E              ;High Byte Stack Pointer Address

.equ     SPL    =$3D              ;Low Byte Stack Pointer Address

.equ     RAMEND =$25F             ;Stack Address

;Interrupt control Addresses

.equ     GIMSK  =$3B              ;General Interrupt Mask Address

.equ     MCUCR  =$35              ;Machine Control Unit Control Register Address

;Port Addresses

.equ     PORTC  =$15              ;Port C Output Address

.equ     DDRC   =$14              ;Port C Data Direction Register Address

;Interrupt Vector Addresses

.equ     INT1addr=$002            ;External Interrupt0 Vector Address

;Register Definitions

.def     leds   =r0               ;Register to store data for LEDs

.def     temp   =r16              ;Temporary storage register

.def     chdir  =r20              ;Register determining sequence direction of LEDs

.org     $0000

rjmp   reset             ;Reset vector

;Set interrupt vectors

.org     INT1addr

rjmp   INT1              ;External Interrupt0 Vector

.org     $0015                     ;Program address

;Program Initialisation

;Set stack pointer to end of memory

reset:   ldi    temp,high(RAMEND)

out    SPH,temp          ;Load high byte of end of memory address

ldi    temp,low(RAMEND)

out    SPL,temp          ;Load low byte of end of memory address

;Initialise output ports

ldi    temp,$ff

out    DDRC,temp         ;Set Port C for output by sending $FF to direction register

;Initialise Interrupt

ldi    temp,$80

out    GIMSK,temp        ;Enable interrupt 0 (INT0)

ldi    temp,$02

out    MCUCR,temp        ;Set Interrupt to occur on a falling edge

;Initialise Main Program

sec                      ;Set carry flag to 1

clr    leds              ;Clear leds

ldi    chdir,$0F         ;Set sequence direction to left

sei                      ;Enable interrupts

;Main Program

forever: out    PORTC,leds        ;Display leds to port C 1

rcall  delay             ;Call delay subroutine

tst    chdir             ;Test if negative

brmi   right             ;If chdir positive sequence right

rol    leds              ;Rotate leds left by 1 bit through carry flag

rjmp   forever

right:   ror    leds              ;Rotate leds right by 1 bit through carry flag

rjmp   forever

;External Interrupt0 Service Routine

INT1:    cli                      ;Prevent any more interrupts while ISR is running

swap   chdir             ;Flip nibbles

reti                     ;Return and enable interrupts again

;delay section of code (25.348 ms @ 1MHz) - utilises r25,r24

delay:   ldi    r24,$21          ;Initialise 2nd loop counter

loop2:   ldi    r25,$FF          ;Initialise 1st loop counter

loop1:   dec         r25              ;Decrement the 1st loop counter

brne   loop1            ;and continue to decrement until 1st loop counter = 0

dec    r24              ;Decrement the 2nd loop counter

brne   loop2            ;If the 2nd loop counter is not equal to zero repeat the 1st loop, else continue

ret                   ;Return

TASK 8

;Program to sequence LEDs on port C, using a look up table

;Stack and Stack Pointer Addresses

.equ     SPH    =$3E              ;High Byte Stack Pointer Address

.equ     SPL    =$3D              ;Low Byte Stack Pointer Address

.equ     RAMEND =$25F             ;Stack Address

;Port Addresses

.equ     PORTC  =$15              ;Port C Output Address

.equ     DDRC   =$14              ;Port C Data Direction Register Address

;Register Definitions

.def     leds   =r0               ;Register to store data pointed to by Z

.def     temp   =r16              ;Temporary storage register

.def     count  =r17

.def     YL     =r28              ;Define low byte of Y

.def     YH     =r29              ;Define high byte of Y

.def     ZL     =r30              ;Define low byte of Z

.def     ZH     =r31              ;Define high byte of Z

;Program Initialisation

;Set stack pointer to end of memory

ldi    temp,high(RAMEND)

out    SPH,temp          ;Load high byte of end of memory address

ldi    temp,low(RAMEND)

out    SPL,temp          ;Load low byte of end of memory address

;Initialise output ports

ldi    temp,$ff

out    DDRC,temp         ;Set Port C for output by sending $FF to direction register

;Main Program

reset:   ldi    ZL,low(table*2)   ;Set Z pointer to start of table

ldi    ZH,high(table*2)

clr    count             ;Set table position counter to zero

next:    rcall  delay             ;Call delay subroutine

lpm                      ;Load R0 with data pointed to by Z

out    PORTC,leds        ;and display data on port C

adiw   ZL,1              ;Increment Z to point to next location in table

inc    count             ;Increment table position counter

cpi    count,14           ;and test if end of table has been reached

brne   next              ;if not the end of the table, get next data value in table

rjmp   reset             ;else reset Z pointer to start of table

;Delay Subroutine (25.349 ms @ 1MHz)

delay:   ldi    YH,high($FFFF)    ;Load high byte of Y

ldi    YL,low($FFFF)     ;Load low byte of Y

loop:    sbiw   Y,1               ;Decrement Y

brne   loop              ;and continue to decrement until Y=0

ret                      ;Return

table:   .DB $3F,$06,$5B,$4F,$66,$6D,$7D,$6F,$77,$7C,$39,$5E,$79,$71